SECTION 09725

VINYL WALL COVERING

PART 1 GENERAL

1.1 REFERENCES

1.2 SUBMITTALS

1.3 DELIVERY, HANDLING, AND STORAGE

1.4 GENERAL INFORMATION

PART 2 PRODUCTS

2.1 VINYL WALL COVERING

2.2 WALL COVERING ADHESIVE, PRIMER, AND SEALER

PART 3 EXECUTION

3.1 INSTALLATION

1 Conditions at Building

2 Preparation of Substrate Surfaces

3 Wall Covering Application

4 Cleaning

# GENERAL

## REFERENCES

#### ASTM INTERNATIONAL (ASTM)

#### ASTM D 751 (2000) Coated Fabrics

## SUBMITTALS

#### Fabrication Drawings shall be submitted for in accordance with paragraph entitled, "General Information," of this section.

## DELIVERY, HANDLING, AND STORAGE

#### Materials shall be delivered to the project site in their original packages or containers bearing labels clearly identifying the manufacturer, brand name, quality or grade, and UL fire-hazard classification when applicable. Materials shall be stored in their original, unbroken packages or containers, in a weathertight and dry place having a temperature not less than 65 degrees F 18 degrees C, for at least 24 hours before the start of wall covering installation.

## GENERAL INFORMATION

#### Fabrication Drawings shall be submitted for vinyl wall covering systems consisting of fabrication and assembly details to be performed in the factory.

# PRODUCTS

## VINYL WALL COVERING

#### Vinyl wall covering shall be an integrally pigmented, opaque vinyl film, laminated to, or combined with, a fabric backing.

## WALL COVERING ADHESIVE, PRIMER, AND SEALER

#### Wall covering adhesive, primer, and sealer shall comply with the vinyl wall covering manufacturer's recommendations.

# EXECUTION

## INSTALLATION

#### Conditions at Building

Spaces to receive vinyl wall covering shall be maintained at a temperature of at least 70 degrees F 21 degrees C at wall base level for at least 48 hours prior to, during, and for 48 hours after the completion of the wall covering installation.

#### Preparation of Substrate Surfaces

Surfaces to receive specified materials shall be clean, smooth, thoroughly cured.

#### Wall Covering Application

Application of adhesive and wall covering shall be in accordance with the wall covering manufacturer's approved descriptive data,

#### Cleaning

Wall covering surface shall be cleaned of excess adhesive and soil in accordance with the wall covering manufacturer's cleaning instructions.