SECTION 09250

GYPSUM BOARD

PART 1 GENERAL

1.1 SUMMARY

1.2 REFERENCES

1.3 QUALITY ASSURANCE

1.4 DELIVERY, STORAGE, AND HANDLING

1.5 PROJECT CONDITIONS

PART 2 PRODUCTS

2.1 MANUFACTURERS

2.2 ACCESSORIES

PART 3 EXECUTION

3.1 INSTALLATION

3.2 PROTECTION

General

## SUMMARY

#### Section Includes:

Wood framing for the support of gypsum board interior.

Accessories for the installation and trimming of gypsum board partitions and ceilings.

#### Related Sections:

Section 06100, Rough Carpentry.

## REFERENCES

#### American Society for Testing and Materials (ASTM):

C 36, Specification for Gypsum Wallboard.

C 1280, Specification for Application of Gypsum Sheathing Board.

C 1396, Specification for Gypsum Board.

#### Gypsum Association (GA):

GA-214, Recommended Specifications: Levels of Gypsum Board Finish.

## QUALITY ASSURANCE

#### Single-Source Responsibility:  Obtain gypsum board products, joint treatment products, and textured coatings from a single manufacturer.

## DELIVERY, STORAGE, AND HANDLING

#### Packaging and Shipping:  Have materials shipped in manufacturer's original packages showing manufacturer's name and product brand name.

#### Storage and Protection:  Store materials inside and protected from damage by the elements.  Protect ends, edges, and faces of gypsum boards from damage.  Protect steel studs and accessories from bending.

## PROJECT CONDITIONS

#### Environmental Requirements:  Establish and maintain application and finishing environment in accordance with ASTM C 840.

# PRODUCTS

## MANUFACTURERS

#### National Gypsum Company:

Gypsum Board:

Regular Board:  Gold Bond Brand Gypsum Wallboard.

Primer: Proform Brand Surfacer/Primer.

## ACCESSORIES

#### Corner Bead:  Formed galvanized steel angle, min. base steel 0.014 in. thick, and complying with ASTM C 1047.

#### Casing Bead:  Formed galvanized steel trim, min. base steel 0.014 in. thick, and complying with ASTM C 1047, Type as follows:

LC-Bead.

L-Bead.

U-Bead.

#### Control Joint:  Extruded vinyl formed with V shaped slot covered with removable flexible vinyl strip and complying with ASTM C 1047.

#### Control Joint: Bent zinc sheet formed with V shaped slot, covered with plastic tape, with perforated flanges and Complying with ASTM C 1047.

# EXECUTION

## INSTALLATION

#### General:  In accordance with the following reference standards and manufacturer's recommendations:

Wood Framing: ASTM C 754.

Gypsum Sheathing Board: ASTM C 1280.

Gypsum Board and Joint Treatment: ASTM C 840.

## PROTECTION

#### Protect gypsum board installations from damage and deterioration until the date of Substantial Completion.