SECTION 07192

VAPOUR BARRIER

PART 1 GENERAL

1.1 RELATED WORK

1.2 REFERENCES

1.3 INSTALLERS QUALIFICATIONS

1.4 QUALITY ASSURANCE

1.5 DELIVERY, STORAGE AND HANDLING

1.6 DURABILITY

PART 2 PRODUCTS

2.1 STANDARD OF ACCEPTANCE

2.2 MEMBRANE MATERIALS

2.3 ACCESSORY

PART 3 EXECUTION

3.1 EXAMINATION

3.2 PREPARATION

3.3 INSTALLATION

3.4 PROTECTION

# GENERAL

## Related Work

#### Wood Framing, blocking, curbs Section 06100

## References

#### Canadian Construction Materials Centre (CCMC), Technical Guide for Air Barrier Systems for Exterior Walls of Low-Rise Buildings

#### Canadian Standards Association, Guideline on Durability in Building CSA S478-95

#### American Society for Testing and Materials (ASTM)

ASTM E 330-[97el], Standard Test Method for Structural Performance of Exterior Windows, Curtain Walls, and Doors by Uniform Static Air Pressure Difference

ASTM E 1186-[98], Standard Practices for Air Leakage Site Detection in Building Envelope and Air Retarder Systems.

#### National Building Code 1995, Part 5, Environmental Separation

## Installers Qualifications

#### The air barrier contractor shall be officially recognized as a Licensed Contractor by NABA.

#### The contractor shall carry liability insurance and bonding.

## Quality Assurance

#### Work performed must be licensed under the NABA Quality Assurance Program.

#### vapour barrier installers must be trained and certified by NECA and PSDI.

#### Testing:

.1 The Designated Applicator and the Certified Installers/Apprentices shall routinely inspect and test their work as they proceed.

.2 The Designated Applicator shall inspect/test all membranes and make all necessary repairs immediately prior to their being permanently covered up.

.3 The Designated Applicator shall cooperate with the air barrier inspector/consultant by making construction drawings and records available to him.

## Delivery, Storage and Handling

#### Materials shall be delivered to the jobsite in manufacturer’s standard commercial containers unopened, undamaged and bearing the name of the manufacturer.

## Durability

#### Product manufacturers must certify in writing that their products will meet all the characteristics required by the Guidelines on Durability in Buildings CSA S478-95.

# PRODUCTS

## Standard of Acceptance

#### Vapour barrier materials which have been evaluated using Technical Guide for Air Barrier Systems for Exterior Walls of Low Rise Buildings, Masterformat Section 07195, produced by CCMC and meet or exceed the properties as defined in section 5.0 Technical Requirements, are acceptable for use.

## Membrane Materials

#### Membrane materials shall be approved by the manufacturer for use on the substrate.

## Accessory

#### Accessory products must be chemically and physically compatible with the materials.

# EXECUTION

## Examination

#### Prior to commencement of work, the installer shall report in writing to the Owner’s representative, any defects in surfaces or conditions, which may adversely affect the performance of products installed under this section

## Preparation

#### Clean and prepare all substrates in accordance with membrane manufacturer’s application guidelines.

## Installation .

#### Install materials to prepared substrate in strict accordance with manufacturer’s instructions.

#### Detail work must be carefully carried out to ensure the vapour barrier membrane creates a continuous seal.

#### Apply transition membrane to prepared surfaces as indicated on drawings and as required by site conditions.

#### The installer shall ensure that the vapour barrier material is structurally supported in all areas including transition areas.

## Protection.

#### Protect finished work. Close up air barrier membranes as soon as possible .