TV NOTCH FILTER

Also, " Snooper" & "Bullet"

© 1997 Info-World, All Rights Reserved.  This information may not be reproduced or transmitted in any form.  Violators will be prosecuted to the fullest extent.

NOTE: Always have the high side of the coils facing the

same direction.

NOTCH FILTER THEORY OF OPERATION

The low frequency version (channel 2 to 6 - 55 MHz to 90 MHz) of this kit has 4 poles.

Each pole has approximately 11 to 14 dB of attenuation at the resonant frequency. The high

frequency version channels 14 to 22 and channels 7 to 13 (100 MHz to 215 MHz) has 3

poles, each pole has approximately 13 to 18 dB of attenuation at the resonant frequency.

The bandwidth (notchwidth) is approximately 300 to 500 kHz for each pole of either filter

design. Since each pole may be adjusted independently the notch depth and width are

somewhat variable and adjustable. If all poles of the Filter are set to the same resonant

frequency the overall depth will be 45 to 55 dB and the notchwidth will be approximately

0.5 MHz. However, if you set each pole of your filter to a different frequency the overall

depth will be approximately 1 5dB and the notchwidth will be 1.0 to 1.5 MHz.

If you want to "trap-out" an unwanted carrier it is normal to adjust all poles to the same

frequency. You could "trap-out" a carrier which has been placed within in a TV channel

between the color-burst signal and the audio carrier, without any appreciable affect on

either of the desired signals. This carrier that you might want to remove normally causes a

annoying beeping sound, and causes the picture to be un-watchable. By removing only the

unwanted carrier you can restore a good picture.

PARTS LIST

NOTE: The capacitors used in this kit may be disk or monolithic.

3ea	2.2 pF cer disk cap (2.2, 2R2@ 229)

lea	3.3 -  cer disk cap (3.3, 3R3, 339)

lea	4.7 pF cer disk (4.7, 479, 4R7)

2ea	5.6 pF cer disk cap (5.6, 5P6, 5R6, 569)

2ea	8.2 pF cer disk cap (8.2, 8R2, 829)

2ea	10	pF cer disk	(marked lOD, 100, 10)

2ea	12	pF cer disk	(marked 12 or 120)

2ea	18	pF cer disk	(marked 18 or 180)

2ea	22	pF cer disk	(marked 22 or 220)

2ea	27	pF cer disk	(marked 27 or 270, 27K)

2ea	33	pF cer disk	(marked 33 or 330)

2ea	56	pF cer disk	(marked 56 or 560)

2ea	5 1/2 turn slug tuned coil (alum core) .16 to .24 uH

3ea	3 1/2 turn slug tuned coil (alum core) .06 to .09 uH

lea	2 1/4 turn air coil 5/32" dia.

lea	1 1/2 turn coil 5/32" dia.

2ea	P C mount 'F' connectors

lea	P C board (080-00212A)

SCHEMATICS

CHART OF FREQUENCIES

4 - POLE FILTER FREQUENCIES

Channel	Video Color           Audio

2	 55.25	 58.83	 59.75

3	 61.25	 64.83	 65.75

4	 67.25	 70.83	 71.25

5	 77.25	 80.83	 81.75

6	 83.25	 86.83	 87.75

Bullet	103 MHz to 105.5 MHz

Snooper	107.997 MHz

CHART OF FREQUENCIES

3 - POLE FILTER FREQUENCIES

Channel	VIDEO	COLOR	AUDIO

7	175.25	178.83	179.75

8	181.25	184.83	185.75

9	187.25	190.83	191.75

10	193 25	196.83	197.75

11	199.25	202.83	203.75

12	205.25	208.83	209.75

13	211 25	214.83	215.75

14	121 25	124.83	125.75

FILTER

15			127.25	130.83	131.75

16			133.25	136.83	137.75

17			139.25	142.83	143.75

18			145.25	148.83	149.75

19			151.25	154.83	155.25

20			157.25	160.83	161.75

21			163.25	166.83	167.75

22			169.25	172.83	173.75

ALIGNMENT

Alignment procedure

for removing an unwanted carrier

on channels 2 to 6

- Adjust L1 so the sound starts to clear.

- Adjust L4 for the best sound the picture the picture should also start to clear.

- Adjust L2 for the best picture and sound.

Adjust L3 several times, the adjustments are critical and 'touchy' be patient

and be careful.

Alignment procedure

for removing an unwanted carrier

on channels 7 to 22

1. Adjust L1 so the sound starts to clear.

2. Adjust L4 for the best sound the picture.

3. Adjust L2 for the best picture and sound.

4. Repeat steps 1 to 3 several times, the adjustments are critical and 'touchy' be patient

and be careful.

Alignment procedure

for

'Snooper' and 'Bullet'

The 'Bullet' and 'Snooper’ are best aligned with a sweep generator or spectrum analyzer.

You must know the exact frequency that is used on your system, and the filter must be

calibrated to that frequency.

BLOCK A CHANNEL

If you want to remove an unwanted channel you would need to use 2 of this filters. One

filter would be adjusted to remove the video information and you would use the other filter

to remove the audio and color information.  most of the video information is contained

within  a 600 kHz bandwidth, therefore one filter will provide approximately 40 dB of

attenuation with a notchwidth of about 600gHz. Typically you will adjust 3 poles of a 4

pole filter or 2 poles of a 3 pole filter for a frequency about l00 kHz above the video

frequency of the desired channel and the remaining pole would be adjusted to a frequency

approximately 400 MHz higher than the video frequency. (see chart of frequencies for

channels 2 to 22).

BULLET; The “BULLET" is a signal used to turn on and off cable services. The signal is

used some systems to remotely activate / de activate the premium services supplied by

the operator without actually having to go to each location.

Snooper; There is a signal inserted in the cable on some systems that is used to detect

cable leaks and unauthorized equipment, to detect this signal you use a device called a

Snooper.

PART SELECTION CHART

PART   CHANNEL  CHANNEL CHANNEL CHANNEL CHANNEL CHANNEL CHANNEL CHANNEL CHANNEL BULL

#	2-3	4	5-6	7-8	9-10	11-13	14-16	17-19	20-22	SNOOP

C1	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2p

C2	33pF	22pF	18pF	8.2pF	8.2pF	5.6pF	18pF	12pF	10pE	27pF

C3	22pF	8.2pF	12pF	5.6pF	4.7pF	4.7pF	4.7pF	3.3pF	2.2pF	8.2p

C4	56pF	56pF	33pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2pF	2.2p

C5	56pF	56pF	33PE	8.2pF	8.2pF	5.6pF	18pF	12pS	10pE	27pF

C6	22pF	8.2pF	12pF	----	----	----	----	----   ----   ----

C7	33pF	22pF	18pF	----	----	----	----	----   ----   ----

C8	2.2pF	2.2pF	2.2pF	----	----	----	----	----   ----   ----

L1	3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3-5Turn 3.5Turn 3.5Turn 3.5Turn 3.5T

L2	5.5Turn 5.5Turn 5.5Turn 5.5Turn 5.5Turn 5-5Turn 5.5Turn 5.5Turn 5.5Turn 5.5T

L3	5.5Turn 5.5Turn 5.5Turn 2 1/4 A 2 1/4 A 1 1/2 A 2 1/4 A 2 1/4 A 2 1/4 A 2 1/4A

L4	3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5Turn 3.5T

J1 & J2	   ----- INSERT AND SOLDER J1 & J2 IN PRINTED CIRCUIT BOARD -----

* NOTE: L3...Some coils inL3 location are marked 2 1/4 A or 1 1/2 A this indicates a

wire wound air coil 5/32" dia. made with enameled wire.

* **** All capacitors and coils must be inserted into printed circuit board so the body

touches the PC board with very short leads.

* **** INSERT ALL PARTS ON FIBER SIDE AND SOLDER ON SILVER SIDE