## INTERIOR INSPECTION

- Battery - On

*Intercom - 3 Out & Up

Steps - Stowed

*Strapped In/Silver Key

*Handgrips - Down/2 Pins

*PRICE Check

*Circuit Breakers - In

Three Outboard

Rudder Trim - Centered

*Throttles - Off

Speed Brake - Open(Aft)

Compass Switch - Mag

*Fuel Shutoff Swtchs - Norm

Alt Release Handle - In

Landing Light - Off

*Gear Hndle - Down/3Grn

*Red710/1.2-1.4/BUG

*Stby ADI - Uncaged/3 Low

*G meter - 3 on 1

*Clock set/Good Sweep

*Nrm/TACAN/Beacon

*Nav/Comm Swtchs - Fwd

Comm Antenna - Auto

*UHF/TACAN/ILS - Set

Mag Compass - Check

ATIS - Check

*Altimeter - Set/Reads

*VVI - Reads

Cabin Altimeter - Check

Ram Dump - Cabin Press

Air Conditioner - Auto

Pitot Heat - Off

Anti-Ice - Off

Boost Pumps - On

Crossfeed - Off

Generators - On

IFF - Squawk Stby

*Warning/Caut Lts - Test

*Interior Lts - Off

Form/Position Lts - Off

Beacon - On

*Loose Items/AOA/Pubs

## ENGINE START

Hands Clear - External Air

Start Button - Push at 14%

Throttle-Idle

Engine Inst - Check

Hyd Pressure - Check

CWI Panel - Check

## AFTER START

*Throttle Gate - Engage

Air-Disconnect

Circuit Breakers - In

Battery - On

## BEFORE TAXI

- Defog/Temp - Set

*G-Suit - Test/Zippers

Yaw Damper - On

Trim - Check

Flt Ctls - Check

Speed Brake - Closed

Flab Slab - Check

*Comm/Nav - Check

180 - Press to Test

270 - 7 Sec/Man Chk

DME - 0.0 +/- 0.5

Bearing - 180 +/- 2

CDI Cnt +/-1/2 Dot

TO/FROM - TO

TACAN/ILS/LOC

IFF/SIF - Check

*Altimeter - Check Stby

Fuel/Oxy Gage - Test

Crossover - Check

*Seat Height - Adjust

*Pins - Stowed

*Brakes - Check

Chocks - Remove

## BEFORE TAKEOFF

*Takeoff Data - Review

Battery Switch - Chk On

Defog/Cabin Temp - Set

Engine Anti-Ice - Set

*Loose Items - Stow

*Helmet Visor - Down

*Flight Controls - Chk

Takeoff Trim  - Press

Canopy - Lock/Light

## LINE UP

- Pitot Heat - On

IFF/SIF - On

Nosewheel Steer - Dseng

Throttles - MIL

Master Caut Light - Out

Engine Inst - Check

Hydraulic Press - Check

## CLIMB

*Oxygen Sys - Check

*Fuel Qty/Balance -Chck

Cabin Press/Defog/Temp

## LEVEL OFF

*Oxygen Sys - Check

*Fuel Qty/Balance

Cabin Press

*Altimeter

## FANG

- Fuel Qty/Balance

Altimeter

NAVAIDS

G-Suit

## DECENT

*Helmet Visor - Down

*Heading/Attitude Sys

*Altimeter

*Fuel Qty/Balance

Crossfeed - Off

Defog & Temp

Pitot Heat/Anti-Ice

Land/Pos Lights - On

## BEFORE LANDING

*Patt Airspeeds - Compute

*Gear - Down

*Hydraulic Press - Check

*Flaps - As required

## AFTER LANDING

*Seat/Canopy Pins - In

Pitot Heat-Off

*Loose Items - Stow

Cabin Altimeter - Check

Cabin Press - Ram Dump

*Canopy - Unlock

Gear Doors - Open

Takeoff Trim - Press

*Wing Flaps - Up

Speed Brake - Open

Landing/Taxi Lts - Off

*TACAN/ILS/IFF/SIF-Off

Cabin Press Switch - CP

# ENGINE SHUTDOWN

- Engine@70%for 1 min

*Seat - Full Up

Form/Position Lights - Off

*Oxygen - 100%

Throttles - Off

Rotating Beacon - Off

*Stby ADI - Cage/Lock

*Unguarded Switches - Off

Wheels-chock

*Silver Key - Secure

Battery - Off

Flight Director Switch - Off