T-38 Need to Know Numbers

Gross weight with fuel and two souls:					12,500 lbs

One engine thrust, MIL and MAX:						2050,2900

Fuel flow for one engine, MAX at rest and Mach 1:				7300, 11,400 lb/hr

Pushing start arms ignition circuit for ?: 					30 sec

One engine norm oil capacity, and air expansion space:			4 qt, 1 qt

Gravity flow guaranteed up to ? alt:						6,000

Fuel cutoff switches operated by ? and cut off fuel in ?:			DC, 1 sec

Fuel quantity error is ? in left and ? in right:					+/- 47, +/- 32

Fuel quantity error in back is ? compared to front:				+/-25

Fuel low level light comes on after ? sec below ? amount:			7.5. 225-275 lbs

Gearbox keeps AC output between ? cycl/sec:				320-480

Gearbox shift occurs at ? range:						65-70%

Voltage of battery is ?V and of DC rectifiers is ?V:				24,28

AC system provides ?/?V power:						115/200

AC generators cut-in between ?:						43-48%

Min battery voltage of ? is required to close batt relay:			18 V

Static inverter provides ? VAC:						115

Left engine provides ? psi for ? system:					3000, utility

Right engine provides ? psi for ? system:					3000, flt control

Hyd low press comes on at ? psi, and goes back out at ? psi:			1500, 1800

Rudder limited to ? deg each side when nosegear out 3/4 or less:		6 deg

Landing gear extension/retraction takes approx ?:				6 sec

Gear horn and warning light come on when gear not down and			210 kts, 10,000

locked and ? kts or less, attitude ? or below, or both throttles below ?:		+/-750 ft, 96%

When decelerating, gear light comes on at ?:					210-180 kts;

When accelerating, gear light may not go out until ?:				240 kts

Landing gear alt release pulled out ? inches, and requires ? sec:		10 in, 15-35 sec

Rear canopy follows front by ? after canopy jettison:				1 sec

Aneroid delays parachute opening until between ?:				15,000 to 11,500

Seat ejection occurs ? after canopy jettison:					0.3 sec

Seat adjustment allows for ? inches of adjustment:				5 in

Fuel for left engine is ? gal serviced/usable and ? lbs usable:			293/286 gal, 1953 lbs

Fuel for right engine is ? gal serviced/usable and ? lbs usable:			305/297 gal, 2029 lbs

Total fuel is ? gal serviced/usable and ? lbs usable:				598/583 gal, 3982 lbs

Number of fuel capacitance probes in fuel bladders:				7

Leave both boost pumps on and crossfeed. on with either fuel <? (S/E):	 	250 lbs

Start button should be engaged at ? RPM, ? minimum:			14%,12% MIN

Ignition must occur before fuel flow reaches ?:				360 lb/hr

Wait at least ? min before attempting another start:				2 min

If EGT doesn't rise w/in ? sec of fuel flow indication, abort start:		12 sec

Allow ? min after power applied before taxiing (for gyros):			3 1/2 min

Flaps should be +/- ? of 60% position:					5%

Nosewheel liftoff is ?:							145 KIAS

Aircraft flies off runway at ?:						160 KIAS

Delay t/o ? min after heavy a/c unless xwind > ? kts:				4 min, 5 kts

Abort takeoff if afterburner doesn't ignite within ? sec:			5 sec

If altimeter and standby alt differ > ? below 10,000 or > ? above

10,000, switch to standby mode:						150,250

Enter pattern at ?:								300 KIAS

Nosewheel tire pressure should be ? psi:					75 psi

Mainwheel tire pressure should be ? psi:					250 psi

Final turn speed is ? plus I knot per 100 lb over 1000 lb fuel:			175 KIAS

Final appr speed is ? plus I knot per 100 lb over 1000 lb fuel:			155 KIAS

Main gear touchdown is ? plus I knot per 100 lb over 1000 lb:			130 KIAS

Hydraulic pressure should not fluctuate below ?:				1500 psi

Each generator is ? phase:							3

Normal fuel line pressure is ? psi; light comes on at or below ? psi:		10, 6 psi

Oxygen light comes on at or below ? liter, may flicker at or below ?:  		1, 3 liters

Engine anti-ice doesn't illuminate when system auto activates below ?:  		65 % RPM

There are ? rotating beacons, located where ?:				2, tail, underside

There are ? position lights, located where ?:					4, tail, underside, wing

Battery energizes DC bus for ?:						10-20 min

If oil pressure is high in cold weather, should be in limits Win ?:		6 min

Holding radio override switch cuts UHF volume by ?:			50%

Warm up time for transponder is ?:						3 min

When IDENT selected, timer energized for ?:				15-30 sec

Normally, precession in ADI won't exceed ? deg in pitch, bank

or heading after over the top maneuvers:					4 deg

Standby Al provides ? deg climb, ? deg dive, ? deg roll capacity,		92, 78, 360 deg,

and provides ? minutes of useful info within +/- ? deg after DC failure: 		9 min, +/- 6 deg

Holding FAST ERECT erects att. gyro at min rate of ? deg/minute:		15 deg/minute

Hold level unaccelerated flight for fast slave for at least ? sec:			30 sec

Allow ? min between actuation of fast slave cycle:				2 min

Each engine has ? dual-element thermocouples:				8

RPM at which T5 system overrides hydromech and opens nozzles:		-97% RPM

Power must be at least ? RPM to ensure effective anti-ice operation:  		80% RPM

Due to oil supply restriction, OG flt restricted to ? sec:			10 sec

Due to oil supply restriction, neg G flt restricted to ? sec:			60 sec'

RPM of ? is the heart of airstart envelope at any altitude:			18-20%

Above ? altitude, avoid rapid throttle movements:				FL 280

Above ? altitude, other than start and level and afterburner initiation

are not recommended:							FL 350

Avoid throttle settings at or above ? if T5 system failed:			97%

Rudder system failure may push rudder to ? deg as quick as ?:			30 deg, 3-5 sec

Stab aug system failure may push rudder to ? deg as quick as ?:		4 deg, 0.25 sec

Minimum util sys press to retract speed brake ?:				250 psi

Plan to touchdown at ? kts above normal touchdown or MCA, whichever >:  	10 kts

Touchdown is possible as high as ? kts:					200 kts

With asymmetric flaps, maintain ? kts above final appr/touchdown speed:	20 kts;

Do not touchdown at less than ? kts with asymmetric flaps:			160 kts

On SE go-around, climb only as necessary until reaching ? kts:			190 kts

When landing with three gear up, touchdown ? kts faster than normal:		10 kts

Max crosswind, dry runway:						30 kts

Max crosswind, dry runway, single ship touch & go:				25 kts

Max crosswind, wet runway:						20 kts

Max crosswind, solo:							15 kts

Max crosswind, icy or standing water:					10 kts

RPM of ? should be sufficient for taxi:					70%

HSL must be within ? deg of mag comp and ? deg within known hdg:		8 deg, 5 deg

Takeoff roll and CFL increase by ? for rolling takeoff:			150-300 ft

Rolling t/o must be single ship, daylight, and runway length ? > CFL:		500 ft

Minimum runway length for takeoff/landing is ?:				8000 ft

Maximum speed for nosewheel on runway:					174 kts

Estimation for landing roll is ? ft + fuel (not including 1000 ft down):		2500 ft

Landing roll approximately ? for no flap landing:				doubled

No flap landings are not accomplished with ? lbs or greater of fuel:		2500 lbs

For practice S/E approach, set down engine at ? rpm:				60%

Lose ? % thrust from anti-ice in MIL:					9%

Taking off behind a full-stop requires ? ft:					6000 ft

Low and normal ? patterns prohibited on initial takeoff.			closed

Low level min airspeed, planned grndspeed, and max airspeed:			300,360,420

Begin closed pullup with min of ? kts, and maintain min of ? kts:		240, 200 kts

Simulated single-engine go-arounds must be initiated by ?:			100 ft AGL

Single-engine go's from full flap overhead must be initiated by ?:		300 ft AGL

Perform all acro and trim maneuvers above ?:				8000 ft AGL

Stall and slow flight must be performed between ?:				8000 ft - FL200

All other airwork min altitude is ?:						8000 ft AGL

Supersonic flt only above ? alt, unless in approved MOA:			FL 300

Solo students should not plan to land with ? fuel or more:			2500lbs

Normal minimum fuel for solo students:					800lbs

Emergency fuel:								500 lbs

Normal recovery fuel for solo students:					1000 lbs

Min taxi interval is ? staggered, ? in trail:					150 ft, 300 ft

Min power setting for stalls and slow flight:					80%

Nose high recoveries must be accomplished below ?:				FL 240

Single-engine full stops can't be done with ?% or less flaps if stopping

distance exceeds ? of available runway:					60% or less, 2/3

Restricted low approach must be initiated by ? unless otherwise directed:	500 ft AGL

Chase aircraft in emergency can low approach down to ?:			300 ft AGL

Gx requires ? G's for ? deg, followed by min of ? G's for ? deg:		4-5 Gs for 90

5 G's for 180

If flab-slab interconnect fails and flaps needed for landing, ? % flaps

provides adequate control authority and controllable stick forces:		30-45%

Pressure reg. maintain cabin alt of ? +/- ? from ? alt.to ? alt:			8000 +/- 2000, 8 to 23

Above FL230, cabin pressure maintains differential pressure

of ? psi above ambient; (Altitude - ?)/2, +/- ?:				5.0 psi, 6000 +/- 2000

Avoid Max thrust dives when below ? lb in either fuel tank:			650

Keep max range airspeed on recovery until ? miles out, then ? airspeed:	 	30 miles, 250 kts

Speed back up to 300 kts ? miles out if going to radar initial:			15 miles

Speed back up to 300 kts ? miles out if going to VFR entry:			5 miles

Use ? kts as go/nogo for simple tire failure:					100 kts

Use highest of which three speeds for go/nogo for engine problems:		ARS, CEFS, DS

Consider using ? speed for catastrophic problems on takeoff:			RS

Increase final approach speed and touchdown speed by ? gust factor:		½

Increase final turn, approach and touchdown by ? kts for no flap:		15 kts

How many seconds until full parachute deployment:				3.42 sec

Best glide speed is ? kts + I knot/ 100 lbs fuel:				230

Multiply altitude by ? for glide distance:					1.6

Landing light retraction takes ? sec:						10 sec

Standby Al erection takes ? :						5 min

3 min for ARU-42/A2

AOA max range is ?, max endurance/best angle of climb is ?:			0.18,0.3

Al warning flag appears for ? min after power is applied:			1 min

AOA fast arrow operative with gear					??

Oxygen hose disconnect pressure:						10-20 lbs

Nozzles should stabilize within ? sec on ground, and ? sec in flight

after rapid throttle movements:						5. 10 sec

Max positive and negative G's fully fueled:					-2.3, +5.6

Max positive G's with 900 lbs or less:					+7.33

Max allowable sink rates for landing are ? fpm normal landing and

9 fpm crab landing (above 1700 lbs fuel); and for < 1700 lbs fuel:		340, 200, 590, 395

First 4 1/2" of stick travel provides ? aileron deflection:			50%

Next 1 1/2" of stick travel provides ? aileron deflection:			Full

Crossfeed recommended with fuel imbalance of ? or greater:			200 lbs

Holding airspeed:								250-265 kts

Radar pattern typically burns ? fuel:					325 lbs

Turbulent air/thunderstorm penetration speed is ? kts:				280 kts

Climb possible at SETOS is ? fpm:						100 fpm.

Climb increased by ? fpm each kt between SETOS - SETOS + 10:		60 fpm

Gear retraction may take ? when retracted between SETOS+ 10 and 190:	1 min

If unable to retract gear, best climb at ? for 60% flaps, ? for 0% flaps:		190 kts, 220 kts

Optimum pitch for ejection zoom:						20 deg

With loss of canopy, slow to ? kts; min drag occurs at ? kts:			300, 225 kts

If one hydr system reads zero, hydr system transfer may occur, and

flight time could be limited to ? min:					35 min

Controllability check should be accomplished at ? AGL or higher:		15,000 ft

Maintain ? kts above min controllable airspeed in descent/landing approach:	20 kts

Pitch and power settings:

Level flight, 300 kts:						90 %,  1100 pph, + 0. 5

Level flight, 250 kts:						87%, 1000 pph. + 1. 5

Enroute descent, 300 kts, 500 ft/nm (X2 w/SB)			80 -5. 0 (X2 W/SB)

Enroute descent, 300 kts, 700 ft/run (X2 w/SB)			IDLE, -7.0 (X2 w/SB)

Level, configured, BugS, 2 engine:					95 % (FF), 90 % (60)

Level, configured, BugS, I engine:					100% (60)

Final approach, configured, 2 engine:				90% (FF)

Final approach, configured, 1 engine:				95%(60)

VVI for precision and non-precision approach:			800; 1200-1500 fpm

Precision approach, full flaps:					90%,-3.0

Precision approach, zero flaps:					89%, +1.0

Non-precision approach, full flaps:					88%,-5.0

Non-precision approach, zero flaps:					87%,-1.0