Collins ADI84A Attitude Direction Indicator

This is another in my series of Collins Avionics instruments. The ADI84A is a multi-purpose attitude and direction indicator incorporating a radar altimeter, V-bars for direction guidance, a glideslope indicator and a localiser deviation pointer, plus the standard slip/skid indicator and pitch and bank indicators. The real-world instrument is also fitted with a decision height light which is coupled to a YG-7500 Radar Altimeter; in this FS98 example the decision height is fixed at 200 feet.

Note: this instrument is not coupled to either my single YG-7500 altimeter or to my FCP65 Flight Director. It is designed as a pure standalone variant.

There are two variants included in this package; ADI84NDH.GAU which has night lighting and ADI84NDHNL.GAU which has no night lighting. Both of them include full and varied failure modes.

Operational Modes

Radar Altimeter

The radalt altitude indicator starts at 2500 feet above the current tile altitude and counts down to zero feet. If you move from a tile at one altitude to another tile at a different altitude while the radalt is showing, the tape will jump to deal with the new tile altitude. Once at ground level the radalt will show a green triangle on a level with the radalt altitude indicator.

Localiser Deviation Pointer

The localiser deviation pointer comes alive when the autopilot has acquired the localiser/glideslope. It moves to show to which side of  the localiser your aircraft currently is; in the screenshot above the approach is off to the starboard side of the runway. The pilot needs to turn to port to correct the approach.

V-bars

The V-bars have two modes of operation, glideslope and navigation. Navigation mode occurs when the aircraft is tracking the current set course; in this mode the V-bars will move left to right across the face of the ADI84A to indicate which direction the aircraft should be turning to follow the set heading. Switching to glideslope mode occurs automatically when the autopilot is set into Approach mode and the glideslope/localiser comes alive. The V-bars will then also move up and down as well as side to side to track the glideslope.

Failure Modes

Electrical Failure will cause a complete shutdown of the instrument with the exception of the pitch/bank indicators. All failure flags will show.

Vacuum Failure will cause the pitch/bank indicators to freeze.

Nav1 Failure will cause the V-bars to retract and the “Gyro Computer” flag will show. GS and Localiser Deviation flags will also show.

Grateful Thanks to:-

Christian Koegler for the explanation of the use of Partial Panel Token Variables in the FS98 SDK. Chris has produced some great gauges both on his own and in conjunction with HGHB Instruments. See www.fly.to/hghb.

Legal Bit:

The undersigned asserts his legal right under UK and US law to be recognised as the author of these gauges. You may use these gauges in any panel of your own. You may also freely distribute these gauges, subject to the no money restriction below and the legal assertion above. You may NOT make money with these gauges in any form without the written and explicit permission of the legal author. You may NOT upload or store these gauges on any WWW site or BBS that charges a fee for access (ISP charges are exempt from this clause). Please do not repaint them - they are an accurate representation of the real thing.

Copyright Dai Griffiths, Dragonflight Design 1999.

-DragonflightDesign@compuserve.com